Lab 001 Single Domain

Load Config

# crpd N/A

Task 1

- Configure a unique, stable router ID on R1 and R2 using their loopback IPs.

- Enable OSPF Area 0 on the R1-R2 link.

- Keep both loopbacks advertised in Area 0 but prevent OSPF adjacency attempts on loopback interfaces by configuring them as passive.

- Set R1's priority on the Area 0 Ethernet link to 255 and R2's priority to 100, then commit the configuration.

- Verify the neighbor state, router IDs, interface, and DR/BDR roles.

Task 2

- Configure the router IDs on R3 and R4.

- Add R1-R3 to Area 1 and R2-R4 to Area 2.

- Force these router-to-router links to OSPF point-to-point network type.

- Configure the loopback on R3 as passive in Area 1 and the loopback on R4 as passive in Area 2.

- Configure the three R4 interfaces facing ep16, ep17, and ep18 as passive OSPF interfaces in Area 2. Do not create OSPF adjacencies with the hosts.

- Verify the neighbors and the OSPF interface types on all routers.

Task 3

Lab 005 Multi Domain

Load Config

# crpd conf load replace /lab-configs/ospf.multidomain.conf

Start from Task 4 BUILD OSPF IN AS 65002