## Prefix Suppression

### Load Config

# IOL configure replace unix:labconfigs/isis.cfg force # IOS-XR configure load config:labconfigs/isis.cfg commit replace confirmed minutes 1 commit

- Configure ISIS so that only the loopbacks are advertised into the IGP.

- Only use interface-level commands on R2 and XR2.

### Answer

#R1 – R6 router isis advertise passive-only #R2 interface Ethernet0/1.12 no isis advertise prefix interface Ethernet0/1.23 no isis advertise prefix interface Ethernet0/1.24 no isis advertise prefix #XR1 router isis 1 address-family ipv4 unicast advertise passive-only #XR2 router isis 1 interface GigabitEthernet0/0/0/0.1920 suppressed

### Explanation