Lab 001 – Junos CLI, Interfaces, VLAN & Access Ports

Objective

- Become comfortable with Junos operational and configuration modes.

- Verify physical and logical interface state.

- Understand VLAN membership, access ports and a basic trunk.

- Verify MAC learning and endpoint connectivity.

- Practice safe Junos configuration workflow with compare/commit/rollback.

Tasks

- Access both SW2 and SW3 and identify hostname, model and Junos version.

- Check interface state.

- Confige VLAN Student and Teacher, access and trunk port.

- Demonstrate failed commit for non-existent vlans configuration.

- Demonstrate show | compare and rollback.

- Demonstrate adding more than 1 vlans on access ports.

- Verify VLAN configuration

- Verify the host-facing port is an access port carrying a single VLAN.

- Verify the inter-switch link is a trunk carrying VLAN Student and Teacher.

- Verify MAC learning on both switches.

- Test end-to-end ping on each VLANs.

- Demonstrate removing Teacher VLAN on the trunk port.

- Demonstrate rollback to the previous state.

Guided Steps

- Access both SW2 and SW3 and identify hostname, model and Junos version.

admin@sw2> show version Hostname: sw2 Model: ex9214 Family: junos Junos: 25.4R1.12 JUNOS OS Kernel 64-bit [20251024.861cae5_builder_bsd15_254] ................ admin@sw3> show system uptime Current time: 2026-10-29 03:12:56 UTC Time Source: LOCAL CLOCK System booted: 2026-10-29 02:55:52 UTC (00:17:04 ago) Protocols started: 2026-10-29 02:57:16 UTC (00:15:40 ago) Last configured: 2026-10-29 02:56:59 UTC (00:15:57 ago) by root 3:12AM up 17 mins, 1 users, load averages: 3.75, 1.69, 0.96 set system time-zone Asia/Jakarta admin@sw3# run show system uptime Current time: 2026-10-29 10:13:31 WIB Time Source: LOCAL CLOCK System booted: 2026-10-29 09:55:52 WIB (00:17:39 ago) Protocols started: 2026-10-29 09:57:16 WIB (00:16:15 ago) Last configured: 2026-10-29 10:13:27 WIB (00:00:04 ago) by admin 10:13AM up 18 mins, 1 users, load averages: 2.95, 1.68, 0.98 admin@sw3> show chassis hardware Hardware inventory: Item Version Part number Serial number Description Chassis VM6AE2B60C6E EX9214 Midplane Routing Engine 0 RE-VMX CB 0 VMX SCB FPC 0 BUILTIN BUILTIN Virtual FPC CPU Rev. 1.0 RIOT-LITE BUILTIN MIC 0 Virtual PIC 0 BUILTIN BUILTIN Virtual

- Check interface state.

admin@sw3> show interfaces ge-0/0/0 Physical interface: ge-0/0/0, Enabled, Physical link is Up Interface index: 143, SNMP ifIndex: 521 Link-level type: Ethernet, MTU: 1514, LAN-PHY mode, Speed: 1000mbps, BPDU Error: None, Loop Detect PDU Error: None, Ethernet-Switching Error: None, Remote Bounce: None, MAC-REWRITE Error: None, Loopback: Disabled, Source filtering: Disabled, Flow control: Enabled, Auto-negotiation: Enabled, Remote fault: Online, Media type: Fiber Device flags : Present Running Interface Specific flags: Internal: 0x101200 Interface flags: SNMP-Traps Internal: 0x4000 Link flags : None CoS queues : 8 supported, 8 maximum usable queues Current address: 52:54:00:12:34:56, Hardware address: 52:54:00:12:34:56 Last flapped : 2026-10-29 09:57:52 WIB (00:32:34 ago) Input rate : 0 bps (0 pps) Output rate : 0 bps (0 pps) Active alarms : None Active defects : None PCS statistics Seconds Bit errors 0 Errored blocks 0 Interface transmit statistics: Disabled Logical interface ge-0/0/0.16386 (Index 333) (SNMP ifIndex 545) Flags: Up SNMP-Traps 0x4004000 Encapsulation: ENET2 Input packets : 0 Output packets: 0 admin@sw3> show interfaces terse ¦ match ge-0/0/1[01] ge-0/0/10 up up ge-0/0/10.16386 up up ge-0/0/11 up up ge-0/0/11.16386 up up

- Configure VLAN Student and Teacher, access and trunk port.

- Demonstrate failed commit for non-existent vlans configuration.

- Demonstrate show | compare and rollback.

- Demonstrate adding more than 1 vlans on access ports.

admin@sw2> configure admin@sw3# set interfaces ge-0/0/0 unit 0 family ethernet-switching vlan members Student [edit] admin@sw3# commit check error: In routing-instance default-switch vlan Student configured under interface ge-0/0/0.0 does not exist error: configuration check-out failed set vlans Student vlan-id 10 set vlans Teacher vlan-id 20 set interfaces ge-0/0/10 unit 0 family ethernet-switching vlan members Student set interfaces ge-0/0/11 unit 0 family ethernet-switching vlan members Teacher set interfaces ge-0/0/2 unit 0 family ethernet-switching interface-mode trunk vlan members [ Student Teacher ] commit check commit admin@sw3# show ¦ compare [edit interfaces ge-0/0/10 unit 0 family ethernet-switching vlan] - members Student; + members [ Student Teacher ]; [edit] admin@sw3# commit check [edit interfaces ge-0/0/10 unit 0 family ethernet-switching] 'vlan' Access interface can be part of only one vlan error: configuration check-out failed

- Verify VLAN configuration

- Verify the host-facing port is an access port carrying a single VLAN.

- Verify the inter-switch link is a trunk carrying VLAN Student and Teacher.

admin@sw3> show vlans brief Routing instance VLAN name Tag Interfaces default-switch Student 10 ge-0/0/10.0* ge-0/0/2.0* default-switch Teacher 20 ge-0/0/11.0* ge-0/0/2.0* default-switch default 1 admin@sw3> show vlans 10 detail Routing instance: default-switch VLAN Name: Student State: Active Tag: 10 Internal index: 3, Generation Index: 2, Origin: Static Mac aging: Enabled MAC aging time: 300 seconds Mac-IP-Snooping Enabled : No VXLAN Enabled : No Interfaces: ge-0/0/10.0*,untagged,access ge-0/0/2.0*,tagged,trunk Number of interfaces: Tagged 1 , Untagged 1 Total MAC count: 2 admin@sw3> show vlans Teacher detail Routing instance: default-switch VLAN Name: Teacher State: Active Tag: 20 Internal index: 4, Generation Index: 3, Origin: Static Mac aging: Enabled MAC aging time: 300 seconds Mac-IP-Snooping Enabled : No VXLAN Enabled : No Interfaces: ge-0/0/11.0*,untagged,access ge-0/0/2.0*,tagged,trunk Number of interfaces: Tagged 1 , Untagged 1 Total MAC count: 2 admin@sw3> show vlans interface ge-0/0/10.0 detail Vlan Name: Student Logical Interface: ge-0/0/10.0 VLAN Index: 3 VLAN ID: 10 Generation: 129 Sequence number: 0 VSTP Index: 12 Handle: 0x6830090 admin@sw3> show vlans interface ge-0/0/2.0 detail Vlan Name: Student Logical Interface: ge-0/0/2.0 VLAN Index: 3 VLAN ID: 10 Generation: 131 Sequence number: 0 VSTP Index: 4 Handle: 0x6830a90 Vlan Name: Teacher Logical Interface: ge-0/0/2.0 VLAN Index: 4 VLAN ID: 20 Generation: 132 Sequence number: 0 VSTP Index: 4 Handle: 0x6830f90

- Verify MAC learning on both switches.

- Test end-to-end ping on each VLANs.

- Demonstrate removing Teacher VLAN on the trunk port.

- Demonstrate rollback to the previous state.

delete interfaces ge-0/0/2 unit 0 family ethernet-switching vlan members Teacher commit rollback 1 commit

Lab 002 – LACP

Objective

- Understand how two physical links form one logical AE interface.

- Configure LACP on both switches.

- Verify AE and member-link status.

- Test redundancy by temporarily disabling one member link.

Tasks

- Check the current configuration of ge-0/0/2 and ge-0/0/3 on both switches.

- Configure ae0 on both switches as trunk ports and allow vlan Student and Teacher.

- Demonstrate that without enabling aggregated ethernet support, ae0 isn’t created.

- Enable support for one aggregated Ethernet interface on SW2 and SW3.

- Configure ae0 for the Layer-2 service required by the lab.

- Assign ge-0/0/2 and ge-0/0/3 as its LACP members.

- Verify ae0 and LACP status on both switches.

- Confirm both physical member links are operational and joined to ae0.Temporarily disable one member link. Verify ae0 remains operational through the remaining link.

- Restore the disabled member link and verify both members return to service.

- Check the final configuration with `show | compare`, commit any pending changes, and verify the final state.

Guided Steps

- Check the current configuration of ge-0/0/2 and ge-0/0/3 on both switches.

admin@sw3> show configuration interfaces ge-0/0/2 unit 0 { family ethernet-switching { interface-mode trunk; vlan { members [ Student Teacher ]; } } } admin@sw3> show configuration interfaces ge-0/0/3

- Configure ae0 on both switches as trunk ports and allow vlan Student and Teacher.

- Configure ae0 for the Layer-2 service required by the lab.

- Demonstrate that without enabling aggregated ethernet support, ae0 isn’t created.

- Enable support for one aggregated Ethernet interface on SW2 and SW3.

# sw2 & sw3 set interfaces ae0 aggregated-ether-options lacp active set interfaces ae0 unit 0 family ethernet-switching interface-mode trunk set interfaces ae0 unit 0 family ethernet-switching vlan members [ Student Teacer ] admin@sw3# run show interfaces ae0 terse error: device ae0 not found set chassis aggregated-devices ethernet device-count 1 admin@sw3# run show interfaces ae0 terse Interface Admin Link Proto Local Remote ae0 up down

- Assign ge-0/0/2 and ge-0/0/3 as its LACP members.

- Verify ae0 and LACP status on both switches.