Table of Content

Table of Content	1

ISIS	1

Prefix Suppression	1

Hello Padding	3

Overload Bit	5

LSP Size	8

Default Metric	9

Hello/Hold Timer	10

Mesh Groups	12

Prefix Summarization	14

Default Route Preference	17

ISIS Timers	20

Log Neighbor Changes	21

Troubleshooting 1 – No routes	23

Troubleshooting 2 – Adjacency	24

IPv6 Single Topology	26

Ipv6 Single Topology Challenge	27

IPv6 Multitopology	29

IPv6 Single to Multitopology	30

Wide Metrics	32

- ISIS

  - Prefix Suppression

    - Load Config

# IOL configure replace unix:labconfigs/isis.cfg force # IOS-XR configure load config:labconfigs/isis.cfg commit replace confirmed minutes 1 commit

- Configure ISIS so that only the loopbacks are advertised into the IGP.

- Only use interface-level commands on R2 and XR2.

    - Answer

# R1 – R6 router isis advertise passive-only # R2 interface Ethernet0/1.12 no isis advertise prefix interface Ethernet0/1.23 no isis advertise prefix interface Ethernet0/1.24 no isis advertise prefix # XR1 router isis 1 address-family ipv4 unicast advertise passive-only # XR2 router isis 1 interface GigabitEthernet0/0/0/0.1920 suppressed

    - Explanation

In ISIS, the link address is not part of the underlying topology. It is simply advertised as an additional prefix in the IP reachability TLV. By configuring advertise passive-only we instruct the router to suppress all prefixes except for the passive interfaces, which in our case is only Lo0.

How does R2 have the correct nexthop for R1’s loopback? We don’t see R1 advertising its nexthop anywhere in the LSP. The answer is that R1 advertises its interface address in the Hello. This is how R2 knows what to use for the nexthop to 1.1.1.1.

  - Hello Padding

    - Load Config

# IOL configure replace unix:labconfigs/isis.cfg force # IOS-XR configure load config:labconfigs/isis.cfg commit replace confirmed minutes 1 commit

- Configure R1 and XR2 so that Hellos are only padded the first six times.

    - Answer

# R1 int Gi2.12 no isis hello padding ! Or router isis no hello padding [multi-point¦point-to-point] # XR2 router isis 1 int Gi0/0/0/0.1920 hello-padding sometimes [level 2]

    - Explanation

By default, the router will always pad Hellos. This can be considered an unnecessary waste of bandwidth, as once you’ve verified MTU via the padded Hellos, you no longer really need to continue to pad Hellos.

When using the above configuration, the router will only pad the first 6 Hellos. After this, all subsequent Hellos are not padded, which saves some bandwidth.

If for some reason you wanted to never pad Hellos, not even at the beginning of the adjacency, you can use no isis hello padding always under the interface on IOS-XE, or hello-padding disable on IOS-XR. This is similar to OSPF’s ip ospf mtu-ignore setting.

When you use no isis hello padding always, you will instead see “Never pad hello PDU”:

On IOS-XR.

XR2 sent Hello without Padding (55 octets), whilst its neighbor (XR1) sent padded Hello (1497 octets).

  - Overload Bit

    - Load Config

# IOL configure replace unix:labconfigs/isis.cfg force # IOS-XR configure load config:labconfigs/isis.cfg commit replace confirmed minutes 1 commit

Set the overload bit on routers R2, R3, and XR1:

- On R2, set overload bit on startup, wait for BGP, and supress all external routes

- On R3, set overload bit right now, and advertise all prefixes

- On XR1, set overload bit on startup, wait for BGP, and suppress both inter-area and external routes

    - Answer

# R2 router isis set-overload-bit on-startup wait-for-bgp suppress external # R3 router isis set-overload-bit # XR1 router isis 1 set-overload-bit on-startup wait-for-bgp

    - Explanation

XR1, we can verify overload bit configuration using show isis protocol.

R3 is currently setting the OL bit in its LSP:

Note that when the OL bit is set, the router is never used for transit, even if no other paths exist. The node can still be used to reach leaf prefixes advertised directly from the node itself, such as its own loopback.

R2 should no longer have ECMP routes to XR1. It should only be using R4 for transit. It will only use R3 for traffic R3 itself is advertising - its loopback and connected routes.

  - LSP Size

    - Load Config

# IOL configure replace unix:labconfigs/isis.cfg force # IOS-XR configure load config:labconfigs/isis.cfg commit replace confirmed minutes 1 commit

- Enable wide metrics everywhere.

- Configure R1 and XR2 so that their LSP is fragmented once is it bigger than 128 bytes.

    - Answer

# IOL router isis metric-style wide # IOS-XR router isis 1 address-family ipv4 unicast metric-style wide # R1 router isis lsp-mtu 128 # XR2 router isis 1 lsp-mtu 128

    - Explanation

By default, an LSP is fragmented at 1500 bytes. We can control this by setting the lsp-mtu. When the router’s own LSP becomes larger than the specified MTU size, it is split into multiple LSPs.

  - Default Metric

    - Load Config

# IOL configure replace unix:labconfigs/isis.cfg force # IOS-XR configure load config:labconfigs/isis.cfg commit replace confirmed minutes 1 commit

- Configure all routers so that the default metric is 100.

    - Answer

# IOL router isis metric-style wide metric 100 # IOS-XR router isis 1 address-family ipv4 unicast metric-style wide metric 100

    - Explanation

To use a default metric of 100, we must use wide metrics. First we enable wide metrics everywhere.

The metric command under the ISIS router process dictates the default metric for all interfaces. Alternatively, you can manually configure the metric under each interface individually.

  - Hello/Hold Timer

    - Load Config

# IOL configure replace unix:labconfigs/isis.cfg force # IOS-XR configure load config:labconfigs/isis.cfg commit replace confirmed minutes 1 commit

- Change the R1-R2 link to be p2p. Configure the Hello/Hold timers to be 2/6 seconds.

- Change the hello/hold timer on R3 so that its Eth0/1.23 interface uses a hold timer of 3 seconds when it is elected DIS on the LAN.

    - Answer

# R1 & R2 interface Ethernet0/1.12 isis network point-to-point isis hello-multiplier 3 isis hello-interval 2 # R3 interface Ethernet0/1.23 isis hello-interval 3 isis hello-multiplier 3

Note : the hello-multiplier command was not strictly necessary above. This is because the default value is 3.

    - Explanation

Before configuring isis network type

Verify holdtime. This is the Neighbor’s Holdtime, not local timer.

We can verify local timer on IOS-XR.

  - Mesh Groups

    - Load Config

# IOL configure replace unix:labconfigs/isis.cfg force # IOS-XR configure load config:labconfigs/isis.cfg commit replace confirmed minutes 1 commit

Configure R2 so that LSPs received on eth0/1.12 are only forwarded out eth0/1.24. LSPs received on eth0/1.23 should only be flooded out eth0/1.24 as well.

    - Answer

# R2 interface Ethernet0/1.12 isis mesh-group 10 interface Ethernet0/1.23 isis mesh-group 10

    - Explanation

By default, an LSP received on any interface will be flooded out all other interfaces. This can be an inefficient use of bandwidth in large scale, highly meshed topologies. To better scale ISIS, we can configure interfaces into groups for flooding.

If an interface belongs to an ISIS mesh-group, any LSP received on that interface is only flooded out other interfaces that are not in the same mesh group.

LSP recv on .12 ├──> .23 ❌ same mesh-group └──> .24 ✅ LSP recv on .23 ├──> .12 ❌ same mesh-group └──> .24 ✅

Before Mesh Group

Shutdown lo0 on R1 to force R1 to update its LSP

After Mesh Group

LSP received on eth0/1.12 is flooded only to eth0/1.24.

  - Prefix Summarization

    - Load Config

# IOL configure replace unix:labconfigs/isis.inter.area.cfg force # IOS-XR configure load config:labconfigs/isis.inter.area.cfg commit replace confirmed minutes 1 commit

The topology has been changed so that there are three ISIS areas now.

- Configure R3 and R4 to summarize R1’s loopback as 1.1.1.0/24 into Level 2.

- Configure R4 to summarize R5’s loopback as 5.5.5.0/24 into Level 1. No other prefixes should be leaked into Level 1.

- Configure XR1 to summarize XR2’s loopback as 20.20.20.0/24 into Level 2.

- Configure XR2 to redistribute a new prefix, 20.20.200.20/32 into ISIS and summarize it as 20.20.200.0/24

    - Answer

# R3 & R4 router isis Summary-address 1.1.1.0 255.255.255.0 level-2 # R4 ip prefix-list L2_To_L1 seq 5 permit 5.5.5.5/32 route-map L2_To_L1 permit 0 match ip address prefix-list L2_To_L1 router isis summary-address 5.5.5.0 255.255.255.0 level-1 redistribute isis ip level-2 into level-1 route-map L2_To_L1 # XR1 router isis 1 address-family ipv4 unicast summary-prefix 20.20.20.0/24 level 2 # XR2 interface Loopback20 ipv4 address 20.20.200.20 255.255.255.255 ! route-policy Lo20 if destination in (20.20.200.20/32) then pass endif end-policy ! router isis 1 address-family ipv4 unicast summary-prefix 20.20.200.0/24 redistribute connected level-1 route-policy Lo20 ! ! end

R1’s Loopback Before / After

R5’s Loopback Before / After

XR2’s Loopback

    - Explanation

When summarizing level 1 prefixes as they are advertised into level 2, you use the command summary-address <prefix> <mask> level-2. Optionally, you can leave the level-2 keyword out, as level 2 is the default.

When summarizing level 2 prefixes as they are advertised into level 1, you must first ensure that the routes are being leaked into level 1 in the first place. Then you use the summary command and specify level-1. The level is always the level you are advertising the prefixes into.

  - Default Route Preference

    - Load Config

# IOL configure replace unix:labconfigs/isis.inter.area.cfg force # IOS-XR configure load config:labconfigs/isis.inter.area.cfg commit replace confirmed minutes 1 commit

- Configure area 49.1234 so that R2 prefers R3 for the default route, and only uses R4 upon loss of R3. Do not use metrics to achieve this.

    - Answer

router isis default-information originate route-map SET_L1 route-map SET_L1 permit 10 set level level-1

    - Explanation

ISIS allows us to originate a default route just like OSPF. An L1 router will prefer an explicit 0/0 route that is advertised in the area over using the ATT bit.

Before Config

After Configuring default-information originate

ISIS RIB still has both 0/0 routes generated from the ATT bit set on R3 and R4, but the explicit 0/0 is preferred and installed.

  - ISIS Timers

    - Load Config

# IOL configure replace unix:labconfigs/isis.inter.area.cfg force # IOS-XR configure load config:labconfigs/isis.inter.area.cfg commit replace confirmed minutes 1 commit

Set R1 and XR2 to use the following ISIS timers:

- Local LSPs should have a max lifetime of 600 seconds. These LSPs should be refreshed every 300 seconds.

- Locally-generated LSPs should be throttled 500 msec after a change, 750 msec if occuring after an LSP was just generated previously, and a max backoff of 3 seconds.

- SPF calculation should be throttled 250 msec after receiving an update, 500 msec if an SPF run has just occured, and a max wait time of 3 seconds.

- Partial route calculation should use the same timers as SPF calculation.

- During flooding, interfaces should pace LSPs at 20 msec intervals

- During flooding, the router should allow for up to 3 seconds before retransmitting the same LSP. If LSPs do need to be retransmitted, these should be paced at 40 msec intervals.

    - Answer

# R1 router isis max-lsp-lifetime 600 lsp-refresh-interval 300 spf-interval 3 250 500 prc-interval 3 250 500 lsp-gen-interval 3 500 750 ! int Eth0/1.12 isis lsp-interval 20 isis retransmit-interval 3 isis retransmit-throttle-interval 40 # XR2 router isis 1 max-lsp-lifetime 600 lsp-refresh-interval 300 lsp-gen-interval initial-wait 500 secondary-wait 750 maximum-weight 3000 ! address-family ipv4 unicast spf-interval initial-wait 250 second-wait 500 maximum-wait 3000 ! int Gi0/0/0/0.1920 lsp-interval 20 retransmit-interval 3 retransmit-throttle-interval 40

    - Explanation

The max-lsp-lifetime command sets the lifetime on locally generated LSPs. By default this appears to be 1200 seconds. This defines how long an LSP is valid in the LSDB. A longer interval allows for less frequent refreshes, which saves some bandwidth in a large network.

  - Log Neighbor Changes

    - Load Config

# IOL configure replace unix:labconfigs/isis.inter.area.cfg force # IOS-XR configure load config:labconfigs/isis.inter.area.cfg commit replace confirmed minutes 1 commit

Configure R1 so that when eth0/1.12 is manually shut down, the following log message appears:

%CLNS-5-ADJCHANGE: ISIS: Adjacency to R2 (Ethernet0/1.12) Down, interface deleted(non-iih)

Configure R2 so that when eth0/1.12 on R1 is shut down, no log message appears at all.

    - Answer

# R1 router isis log-adjacency-changes all # R2 router isis no log-adjacency-changes

    - Explanation

By default, log-adjacency-changes is enabled. However, this only logs adjacency changes that are detected through the ISIS Hello. If the interface goes down, the adjacency is lost based on the interface, not the missing Hellos, so no log is generated. If we use the all keyword, it will log all adjacency changes, even those caused by non-Hello events.

  - Troubleshooting 1 – No routes

    - Load Config

# IOL configure replace unix:labconfigs/isis.tshoot1.cfg # IOS-XR configure load config:labconfigs/isis.tshoot1.cfg commit replace confirmed minutes 1 commit

R1 and XR2 have no ISIS routes in the routing table. Fix the issue without disabling any features that have been enabled on any other routers. You may only configure two routers.

    - Answer

# R1 router isis metric-style wide / transition # XR2 router isis 1 address-family ipv4 unicast metric-style wide / transition

    - Explanation

If we look at the topology on R1 and XR2, we can see that all routers have ** for the metric. This indicates a metric-style mismatch:

  - Troubleshooting 2 – Adjacency

    - Load Config

# R1, R2 configure replace unix:labconfigs/isis.tshoot2.cfg

Fix the broken adjacency between R1 and R2.

    - Answer

Cisco explains that in IPv6 single-topology IS-IS, routers must have the same set of address families on the same link, and IS-IS performs a consistency check on the IPv6 information within the hello packets. If one side runs IPv4+IPv6 while the other side only runs IPv4, the adjacency can be rejected or encounter issues.

R1 R2 E0/1.12 ─────────────────── E0/1.12 IPv4 IS-IS ✅ IPv4 IS-IS ✅ IPv6 IS-IS ✅ IPv6 IS-IS ❌ IPv6 link-local ✅? IPv6 link-local ❌

alternative option for transition

# R1 router isis address-family ipv6 no adjacency-check

    - Explanation

There are several things wrong here. The following must match for an adjacency to come up:

- Common subnet for IPv4

- Protocols supported must match and multitopology mode must match

  - However, if one side uses ST and one uses MT transition, then both IPv4 and IPv6 will work

- Circuit types and router levels must match/be compatible

- Hello auth must match

- MTU must match

The following was broken in this situation:

- R2 has a misconfigured key-string

- R2 is in the wrong IPv4 subnet

- R1 does not use p2p as the circuit type

- R1 is level-1 only, while R2 is level-2 only

- R1 has a larger MTU on the interface. R2 won’t be able to accept fully padded Hellos from R1

- R1 is running IPv6 on the interface and R2 is not

  - IOS-XE uses single-topology by default, in which both IPv4 and IPv6 are required if one side has IPv6 enabled.

  - An alternative would be to enable wide metrics on both sides, and then use multi-topology on R1. This would allow the adjacency to come up without R2 supporing IPv6. This is because each protocol has its own topology now on R1.

  - IPv6 Single Topology

    - Load Config

# IOS-XE config replace unix:labconfigs/isis.ipv6.init.cfg # IOS-XR configure load config:labconfigs/isis.ipv6.init.cfg commit replace y

All routers have IPv6 link-local addresses enabled on their interfaces, and a loopback IPv6 address of 2001:db8::<router num>/128.

- Without enabling wide metrics, configure IPv6 reachability between all routers on their loopbacks.

    - Answer

IOS-XE’s default IPv6 mode is single topology. In single topology, IPv6 “uses” the IPv4 topology. The IPv6 prefixes ride on top of the IPv4 topology and its adjacencies. IPv6 prefixes are simply advertised with TLV 236. This is used whether or not wide metrics are used. (Either can be used with single topology).

The link-local address, used for the nexthop by the neighbor for IPv6 routes, is advertised in the Hello in the IPv6 interface address TLV (232).

On IOS-XR, we must enable single-topology under the AFI. By default IOS-XR uses multi topology, in which IPv4 and IPv6 each have a separate topology, and IPv6 is not required to be enabled on every interface.

In summary:

- IPv6 can be used with either single topology or multi topology

  - In single topology, IPv6 uses the IPv4 topology. IPv6 prefixes are advertised in a separate IPv6 reachability TLV (236)

- Single topology is the default mode on IOS-XE

- Multi topology is the default mode on IOS-XR

- In single topology, every link must have both IPv4 and IPv6. If any router does not have IPv6 enabled, both IPv4 and IPv6 reachability is broken.

  - This is because the adjacency requires all protocols be supported between both neighbors

  - The exception is that you can use single topology and only IPv6. No IPv4 anywhere in the topology. In this case, the protocols supported TLV lists only IPv6 by itself.

- Single topology can use either narrow or wide metrics. Multi topology requires wide metrics.

  - Ipv6 Single Topology Challenge

    - Load

# IOS-XE config replace unix:labconfigs/isis.st.challenge.init.cfg # IOS-XR configure load config:labconfigs/isis.st.challenge.init.cfg commit replace y

- R1 and XR2 have enabled IPv6 support using ST mode.

- R2 and XR1 have not yet implemented IPv6.

- The operator has found that the adjacencies between R1-R2 and XR1-XR2 are now broken.

- Find a way to fix this without enabling IPv6 support on R2 or XR1, and without disabling IPv6 support on R1 or XR2.

    - Answer

# R1 router isis address-family ipv6 no adjacency-check # XR2 router isis 1 address-family ipv6 unicast adjacency-check disable ! !

Explanation

A router that enables IPv6 on an interface using ST mode requires that the neighbor supports both IPv4 and IPv6 in the protocols supported TLV. If the neighbor only supports IPv4, the router will not bring the adjacency up.

To get around this, we can configure the router to disable this adjacency check.

IPv6 Multitopology

Load Config

# IOS-XE config replace unix:labconfigs/isis.ipv6.init.cfg # IOS-XR configure load config:labconfigs/isis.ipv6.init.cfg commit replace y

All routers have IPv6 link-local addresses enabled on their interfaces, and a loopback IPv6 address of 2001:db8::<router num>/128.

Configure IPv6 reachability using multi topology.

Answer

# IOS-XE router isis metric-style wide address-family ipv6 multi-topology int xxx ipv6 router isis # IOS-XR router isis 1 address-family ipv4 unicast metric-style wide ! interface Loopback0 address-family ipv6 unicast ! ! interface GigabitEthernet0/0/0/0.1920 address-family ipv6 unicast

Explanation

show isis ipv6 topology show isis topology # ios-xr show isis database verbose ¦ utility egrep "NLPID¦00-00" # ios-xe show isis database verbose ¦ in NLPID¦00-00

IPv6 Single to Multitopology

Load Config

# IOS-XE config replace unix:labconfigs/isis.ipv6.st.cfg # IOS-XR configure load config:labconfigs/isis.ipv6.st.cfg commit replace y

All routers have IPv6 link-local addresses enabled on their interfaces, and a loopback IPv6 address of 2001:db8::<router num>/128.

Single-topology IPv6 has already been setup. Migrate to Multi-topology for IPv6 without breaking IPv4 or IPv6 reachability.

Explanation

In phase one, we first enable wide-metrics everywhere. This is because MT on IOS-XE requires wide metrics. We enable it using the transition method, in which both narrow and wide metrics are generated and received.

# IOS-XE router isis Metric-style transition # IOS-XR Router isis 1 address-family ipv4 Metric-style transition

Now we can remove the transition style and use only wide-metrics.

# IOS-XE router isis metric-style wide # IOS-XR Router isis 1 address-family ipv4 Metric-style wide

In phase two we enable multi-topology on all routers but with the transition keyword. This forces the router to continue generating the IPv6 reachability TLV which is used in single-topology.

# IOS-XE router isis add ipv6 multi-topology transition # IOS-XR router isis 1 address-family ipv6 no single-topology !!! no transition mode on IOS-XR

Note that we lost reachability on XRv neighbors with this step because we had to enable multi-topology only. There is no “transition” mode in which XRv continues generating normal ST IPv6 prefix reachability TLVs. Specifically, when making the change first on routers R1-R6, and then on XR1, XR1 loses its reachability to XR2 prefixes. But it still has reachability to all routers R1-R6. We then turn ST off on XR2, and reachability is restored.

Now that all routers are running MT, we can turn the transition mode off.

# IOS-XE router isis add ipv6 multi-topology

Summary

The transition keyword for multi-topology is only available on IOS-XE. When you use this keyword, the router continues generating IPv6 reachability TLVs (236) in addition to the MT IPv6 reachable prefixes TLV (237). This allows other routers that are still running in ST mode to continue to work.

Wide Metrics

Load Metric

# CSR configure replace tftp://10.0.0.2/configs/isis.inter.area.cfg # IOS-XR configure load config:labconfigs/isis.inter.area.cfg commit replace confirmed minutes 1 commit

Narrow metric TLVs is 6 bits, so the max value is 63.

Redistribute a route on R1 into level 1 and notice the behavior.

# R1 int lo100 ip add 100.1.1.1 255.255.255.255 ! route-map LO100 match int lo100 ! router isis redistribute connect route-map LO100 level-1

# R1 #R1 router isis metric-style transition

Route Filtering

Load Config

# IOS-XE config replace unix:labconfigs/isis.inter.area.cfg # IOS-XR configure load config:labconfigs/isis.inter.area.cfg commit replace y

On XR2, configure Lo20 with IP 200.20.20.20/32. Redistribute this into ISIS with tag=20.

On R3 and R4, filter routes with tag=20 from being redistributed into L1. Allow all other L2 routes to be redistributed.

  - Answer

First we must switch to wide-style metrics to use tags. The narrow metric external IP reachability TLV does not have a tag field.

Redistribute with correct tag

# XR2 int Lo20 ip add 200.20.20.20/32 ! route-policy lo20 if destination in ( 200.20.20.20/32 ) then set tag 20 endif end-policy ! router isis 1 address-family ipv4 unicast redistribute connected level-1 route-policy lo20

On R3 and R4 we redistribute all L2 routes into L1 except for tag 20:

# R3, R4 route-map nolo20 deny 5 match tag 20 route-map nolo20 permit 10 ! router isis redistribute isis ip level-2 into level-1 route-map nolo20

Backdoor Link

Load Config

# IOS-XE config replace unix:labconfigs/isis.inter.area.cfg # IOS-XR configure load config:labconfigs/isis.inter.area.cfg commit replace y

Configure an additional link between R1 and XR2, using vlan 120 with subnet 20.1.20.0/24.

Configure this link so that area 49.1234 and 49.1920 can reach each other directly without traversing the L2 backbone.

Do not let traffic destined for any other areas run over this link.

Answer

# R1 router isis is-type level-1-2 backdoor # IOS-XR configure load config:labconfigs/isis.inter.area.cfg commit replace y

Explanation

The backdoor feature configures a L1/L2 router to do two things:

- Not set the ATT bit in L1 LSPs

- Continue installing a default route via other L1 routers that have the ATT bit set

On IOS-XE this is configured with is-type level-1-2 backdoor. On IOS-XR this is configured using attached-bit send never-set.