LAB-004

Routing Tables and Next-Hop Resolution

JNCIE-SP Engineer Roadmap · Phase 1: Junos Core Skills · Foundation Track

OBJECTIVE Trace a service route across inet.0, inet.3, mpls.0, protocol-learned routes, the resolver, and the forwarding table; diagnose inactive and hidden routes from evidence.

IMPORTANT OSPF, LDP/MPLS, and iBGP are baseline transport for observation. Preserve the supplied startup configurations. All controlled changes must be removed before final verification.

| Difficulty | Intermediate |
| --- | --- |
| Estimated time | 2 hours |
| Dependency | LAB-003 |
| Devices | R1, R2, R3, R4, client1, server1 |
| Credentials | admin / admin@123 |
| Virtualization status | FULL |

Interface and Addressing Reference

| Node | Interface | Peer interface | IPv4 address |
| --- | --- | --- | --- |
| client1 | eth1 | R1 ge-0/0/0 | 192.168.40.10/24 |
| R1 | ge-0/0/0 | client1 eth1 | 192.168.40.1/24 |
| R1 | ge-0/0/1 | R2 ge-0/0/0 | 10.0.12.1/30 |
| R2 | ge-0/0/0 | R1 ge-0/0/1 | 10.0.12.2/30 |
| R2 | ge-0/0/1 | R4 ge-0/0/0 | 10.0.24.1/30 |
| R4 | ge-0/0/0 | R2 ge-0/0/1 | 10.0.24.2/30 |
| R1 | ge-0/0/2 | R3 ge-0/0/0 | 10.0.13.1/30 |
| R3 | ge-0/0/0 | R1 ge-0/0/2 | 10.0.13.2/30 |
| R3 | ge-0/0/1 | R4 ge-0/0/1 | 10.0.34.1/30 |
| R4 | ge-0/0/1 | R3 ge-0/0/1 | 10.0.34.2/30 |
| R4 | ge-0/0/2 | server1 eth1 | 198.51.100.1/24 |
| server1 | eth1 | R4 ge-0/0/2 | 198.51.100.10/24 |
| R1–R4 | lo0.0 | — | 10.255.4.1–10.255.4.4/32 |

Lab Files

lab004.clab.yml — Containerlab definition

configs/r1.conf through configs/r4.conf — hierarchical startup configurations

verify.sh — container, protocol, RIB, resolver, and end-to-end checks

Task 1 — Deploy and Prove the Baseline

OBJECTIVE Establish a trusted baseline before interpreting route-selection or resolution behavior.

Task

Deploy the topology, run the automated verifier, and prove that OSPF, LDP, iBGP, and bidirectional client-to-server forwarding are healthy. Do not begin the experiments unless the verifier reports PASS.

Solution

1. Deploy from the LAB-004 directory and run the verifier.

sudo containerlab deploy -t lab004.clab.yml chmod +x verify.sh ./verify.sh

2. Log in to R1 and confirm the shortest line.

ssh admin@172.31.4.11 show interfaces descriptions show ospf neighbor show ldp session show bgp summary

3. Prove the complete data path in both directions from the Linux shell.

docker exec client1 ping -c 5 198.51.100.10 docker exec server1 ping -c 5 192.168.40.10

EXPECTED RESULT verify.sh ends with RESULT: PASS. R1 has two Full OSPF neighbors, two operational LDP sessions, an Established iBGP session to R4, and bidirectional host reachability.

EXPLANATION A clean baseline prevents later hidden-route, preference, or label observations from being confused with startup failure or incomplete protocol convergence.

Task 2 — Map the Routing and Forwarding Tables

OBJECTIVE Identify which table answers each control-plane or forwarding question.

Task

On R1, inventory inet.0, inet.3, mpls.0, BGP-learned routes, and the IPv4 forwarding table. For each command, record the route key, protocol, preference, selected next hop, and label operation when present.

Solution

1. Start with table scale and protocol ownership.

show route summary show route table inet.0 protocol ospf show route table inet.3 protocol ldp show route table mpls.0 protocol ldp show route protocol bgp

2. Compare a RIB lookup with the installed IPv4 forwarding entry.

show route 198.51.100.0/24 exact detail show route forwarding-table destination 198.51.100.0/24

IMPORTANT inet.0 is an IPv4 unicast RIB. inet.3 holds usable LSP egress routes for protocol next-hop resolution. mpls.0 is keyed by incoming labels and describes label forwarding actions. The forwarding table is the installed data-plane view.

EXPECTED RESULT R1 learns 198.51.100.0/24 through BGP in inet.0, has LDP routes to remote loopbacks in inet.3, and has locally significant label entries in mpls.0.

EXPLANATION The same packet path is represented differently at each stage: service NLRI in inet.0, transport resolution in inet.3, incoming-label behavior in mpls.0, and the programmed forwarding action in the FIB.

Task 3 — Trace the Service Route Through Resolution

OBJECTIVE Follow a protocol next hop until it becomes a directly usable forwarding next hop.

Task

Trace 198.51.100.0/24 on R1 from the BGP route in inet.0 to protocol next hop 10.255.4.4, the contributing LDP route in inet.3, and the installed forwarding action. Prove the result with traffic.

Solution

1. Inspect the service route deeply enough to expose protocol next-hop and indirect next-hop fields.

show route 198.51.100.0/24 exact extensive

2. Compare the IGP and MPLS resolution candidates for the BGP protocol next hop.

show route 10.255.4.4/32 exact detail show route table inet.3 10.255.4.4/32 exact extensive

3. Inspect the final forwarding entry and verify traffic.

show route forwarding-table destination 198.51.100.0/24 # Linux shell docker exec client1 traceroute -n 198.51.100.10 docker exec client1 ping -c 5 198.51.100.10

IMPORTANT Protocol next hop and forwarding next hop are not synonyms. BGP carries 10.255.4.4 as the protocol next hop; recursive resolution selects a directly usable interface/neighbor and may add an MPLS push operation.

EXPECTED RESULT The active BGP route points indirectly to 10.255.4.4. Its forwarding resolution uses the LDP entry in inet.3 and sends traffic through the lower-cost R1–R2–R4 path.

EXPLANATION BGP asks the resolver to turn a remote protocol next hop into a forwarding next hop. With equal usable resolution candidates, Junos prefers inet.3 and programs the LSP action into the forwarding chain.

Task 4 — Prove Active and Inactive Route Selection

OBJECTIVE Separate candidate-route visibility from active-route and FIB selection.

Task

Create a competing static discard route for 198.51.100.0/24. First make it less preferred than BGP, then more preferred. Predict the active RIB entry and traffic result before each commit. Remove the static route afterward.

Solution

1. Add a less-preferred candidate. BGP should remain active.

configure set routing-options static route 198.51.100.0/24 discard set routing-options static route 198.51.100.0/24 preference 200 show ¦ compare commit check commit comment "LAB004 add inactive static candidate" run show route 198.51.100.0/24 exact detail

2. Lower the static preference value so it wins, using a safety timer.

set routing-options static route 198.51.100.0/24 preference 150 show ¦ compare commit confirmed 10 run show route 198.51.100.0/24 exact detail run show route forwarding-table destination 198.51.100.0/24

3. Prove the forwarding impact, then remove the experiment before the timer expires.

# Linux shell: this ping is expected to fail while discard is active docker exec client1 ping -c 3 -W 2 198.51.100.10 # R1 configure delete routing-options static route 198.51.100.0/24 commit check commit comment "LAB004 restore BGP service route" exit show route 198.51.100.0/24 exact detail

EXPECTED RESULT At preference 200, Static is visible but inactive and BGP/170 remains active. At preference 150, Static becomes active, the FIB changes to discard, and the ping fails. After cleanup, BGP and traffic recover.

EXPLANATION A route can exist in the RIB without being selected. Only the active usable route normally contributes to the forwarding table, so a control-plane preference change can produce an immediate traffic change.

Task 5 — Diagnose a Hidden Recursive Static Route

OBJECTIVE Use hidden-route evidence to distinguish route existence from next-hop usability.

Task

Create 203.0.113.0/24 with an indirect next hop that has no resolving route. Prove why the route is hidden, add the minimum resolver route to activate it, and then remove both test routes.

Solution

1. Create the unresolved recursive route and inspect its state.

configure set routing-options static route 203.0.113.0/24 next-hop 192.0.2.44 set routing-options static route 203.0.113.0/24 resolve commit check commit comment "LAB004 create unresolved recursive route" exit show route 203.0.113.0/24 exact extensive show route hidden 203.0.113.0/24 exact extensive show route resolution unresolved

2. Add only the missing resolver route and recheck the route.

configure set routing-options static route 192.0.2.44/32 next-hop 10.0.12.2 commit check commit comment "LAB004 add resolver route" exit show route 192.0.2.44/32 exact detail show route 203.0.113.0/24 exact extensive

3. Remove the complete experiment.

configure delete routing-options static route 203.0.113.0/24 delete routing-options static route 192.0.2.44/32 commit check commit comment "LAB004 remove recursive-route experiment"

IMPORTANT The resolve option permits a static route to use an indirect next hop; it does not invent reachability. A contributing route to 192.0.2.44 is still required.

EXPECTED RESULT Before the resolver route exists, 203.0.113.0/24 is unusable and appears as hidden/unresolved. After 192.0.2.44/32 is installed through 10.0.12.2, the recursive route becomes active.

EXPLANATION Hidden is a usability state, not proof that the route was never learned or configured. Extensive output should identify an unusable next hop; the minimal repair is to restore the missing resolution chain.

Task 6 — Validate Qualified Next-Hop Failover

OBJECTIVE Control primary and backup static paths with per-next-hop preference and verify FIB failover.

Task

Override the BGP service route temporarily with a static route whose primary next hop is R2 and qualified backup is R3. Fail the primary R1–R2 interface under commit-confirmed protection, prove the backup becomes active without losing server reachability, then fully restore the baseline.

Solution

1. Install the preferred and qualified next hops.

configure set routing-options static route 198.51.100.0/24 next-hop 10.0.12.2 set routing-options static route 198.51.100.0/24 preference 150 set routing-options static route 198.51.100.0/24 qualified-next-hop 10.0.13.2 preference 160 commit check commit comment "LAB004 qualified next-hop experiment" exit show route 198.51.100.0/24 exact extensive show route forwarding-table destination 198.51.100.0/24

2. Fail the primary interface and prove the alternate next hop is selected.

configure deactivate interfaces ge-0/0/1 show ¦ compare commit check commit confirmed 10 exit show interfaces ge-0/0/1 terse show route 198.51.100.0/24 exact extensive show route forwarding-table destination 198.51.100.0/24 # Linux shell docker exec client1 ping -c 5 198.51.100.10

3. Restore the link, then remove the static experiment so BGP owns the service route again.

configure rollback 1 commit check commit comment "LAB004 restore R1-R2 primary link" delete routing-options static route 198.51.100.0/24 commit check commit comment "LAB004 restore BGP route ownership" exit show route 198.51.100.0/24 exact detail

EXPECTED RESULT The static route initially forwards through 10.0.12.2. When ge-0/0/1 is unavailable, 10.0.13.2 becomes the active qualified next hop and traffic continues. Final state returns to BGP/170.

EXPLANATION The route-level preference applies to the primary next hop; the qualified-next-hop preference overrides it only for that alternate. Lower numeric preference is more preferred among usable candidates.

Task 7 — Observe BGP Resolution Fallback from inet.3 to inet.0

OBJECTIVE Change the contributing transport RIB without tearing down the BGP session or IP reachability.

Task

On R1, remove both LDP adjacencies temporarily while leaving OSPF and iBGP operational. Compare the BGP route and forwarding action before and after inet.3 loses the R4 LDP route. Restore LDP and prove MPLS resolution returns.

Solution

1. Capture the labeled baseline.

show route table inet.3 10.255.4.4/32 exact extensive show route 198.51.100.0/24 exact extensive show route forwarding-table destination 198.51.100.0/24

2. Remove LDP from both R1 core links under a safety timer; do not deactivate OSPF or the physical interfaces.

configure deactivate protocols ldp interface ge-0/0/1.0 deactivate protocols ldp interface ge-0/0/2.0 show ¦ compare commit check commit confirmed 10 exit show ospf neighbor show ldp session show bgp summary show route table inet.3 10.255.4.4/32 exact show route 198.51.100.0/24 exact extensive show route forwarding-table destination 198.51.100.0/24

3. Confirm IP fallback still forwards, then restore the previous configuration.

# Linux shell docker exec client1 ping -c 5 198.51.100.10 # R1 configure rollback 1 commit check commit comment "LAB004 restore LDP resolution" exit show ldp session show route table inet.3 10.255.4.4/32 exact extensive show route 198.51.100.0/24 exact extensive

IMPORTANT The intended observation is resolution fallback, not a hidden BGP route. When no usable inet.3 LSP exists, Junos can resolve the BGP protocol next hop through inet.0 and keep traffic alive over native IP.

EXPECTED RESULT OSPF and iBGP remain up. The inet.3 LDP route disappears, but 198.51.100.0/24 remains active and resolves through inet.0. After rollback, LDP sessions and the MPLS forwarding action return.

EXPLANATION BGP evaluates eligible resolution routes in inet.0 and inet.3. An LSP can be preferred for forwarding without being required for basic reachability; removing it exposes the IP resolution fallback.

Task 8 — Correlate Labels and Close the Lab

OBJECTIVE Connect an ingress push action to the transit mpls.0 entry, then finish with a clean and auditable state.

Task

Record the outgoing label used by R1 for 10.255.4.4, locate that incoming label on the primary transit router R2, identify its label operation, and complete the final evidence checklist. Do not assume label values from this guide; labels are allocated dynamically.

Solution

1. On R1, record the label pushed toward R2 for the R4 loopback.

show route table inet.3 10.255.4.4/32 exact extensive

2. On R2, substitute the recorded numeric label and inspect its locally significant mpls.0 entry.

show route table mpls.0 label <recorded-label> detail show route table mpls.0 protocol ldp show route forwarding-table label <recorded-label>

3. Complete the final health and cleanup checks.

# R1 show system commit ¦ last 12 show configuration routing-options static show interfaces descriptions show ospf neighbor show ldp session show bgp summary show route 198.51.100.0/24 exact extensive show route table inet.3 10.255.4.4/32 exact extensive show route hidden # Linux shell docker exec client1 ping -c 5 198.51.100.10 ./verify.sh

IMPORTANT MPLS labels are local to the allocating router. The label R1 pushes is interpreted as an incoming label by R2; it is not a network-wide identifier. Do not paste the angle brackets in the commands.

EXPECTED RESULT R1 shows a Push operation toward R2. R2 finds the same numeric label in mpls.0 and normally performs Pop on the two-hop primary LSP to R4. No test static routes remain, protocol sessions are healthy, traffic succeeds, and verify.sh reports RESULT: PASS.

EXPLANATION inet.3 answers how ingress traffic enters an LSP; mpls.0 answers what a router does when that locally allocated label arrives. Correlating both views completes the RIB-to-label-to-FIB chain.

COMPLETION STANDARD The learner can distinguish active, inactive, and hidden routes; explain protocol versus forwarding next hops; trace BGP resolution through inet.3 or inet.0; correlate label actions in mpls.0; and restore every controlled change with evidence.

Official References

LSP routes and inet.3/mpls.0 behavior: https://www.juniper.net/documentation/us/en/software/junos/mpls/topics/topic-map/lsp-routes.html

Static routes: https://www.juniper.net/documentation/us/en/software/junos/static-routing/topics/topic-map/config_static-routes.html

Static preferences and qualified next hops: https://www.juniper.net/documentation/us/en/software/junos/static-routing/topics/topic-map/static-route-prefer-qualified-next-hop.html

show route hidden: https://www.juniper.net/documentation/us/en/software/junos/cli-reference/topics/ref/command/show-route-hidden.html

BGP next-hop resolution overview: https://www.juniper.net/documentation/us/en/software/junos/bgp/topics/topic-map/bgp-overview.html

LDP overview: https://www.juniper.net/documentation/us/en/software/junos/mpls/topics/topic-map/ldp-overview.html

LAB STATUS FULL — all required route-table, resolver, label, controlled-fault, recovery, and end-to-end validation objectives can be demonstrated on vJunos-router 25.4R1.12.