Table of Content

Table of Content	1

ISIS	1

Prefix Suppression	1

Hello Padding	3

Overload Bit	5

LSP Size	8

Default Metric	9

Hello/Hold Timer	11

Mesh Groups	12

Prefix Summarization	14

Default Route Preference	17

ISIS Timers	20

Log Neighbor Changes	21

Troubleshooting 1 – No routes	23

Troubleshooting 2 – Adjacency	24

IPv6 Single Topology	26

Ipv6 Single Topology Challenge	27

IPv6 Multitopology	29

IPv6 Single to Multitopology	30

Wide Metrics	32

Route Filtering	34

Backdoor Link	35

Non-Optimal Intra-Area Routing	37

Multi Area	38

- BGP

  - Lab1 ECMP

    - Load Config

# IOL configure replace unix:labconfigs/lab1.init.cfg force

The idea behind these labs is that R6 is advertising 192.0.2.6/32 and 2001:db8::6/128. It is multi-homed to R4 and R5. We will use various techniques to achieve load balancing and failover.

This first lab requires you to configure ECMP.

Currently R2 and R3 are using R4 for egress due to its lower RID. R6 is using R4 for this reason as well.

Configure the fewest number of routers so that traffic from R1 to R6 is load balanced through R4 and R5, and return traffic from R6 is likewise load balanced.

Answer

# R3 router bgp 65000 address-family ipv4 maximum-paths ibgp 2 address-family ipv6 maximum-paths ibgp 2 # R6 router bgp 65006 address-family ipv4 maximum-paths 2 address-family ipv6 maximum-paths 2

Explanation

Lab2 UCMP

Load Config

# IOL configure replace unix:labconfigs/lab2.init.cfg force

The bandwidth of the R4-R6 and R5-R6 link has been changed, matching the diagram above.

- Configure UCMP so that R3 will send twice as much traffic to R4.

- Additionally, configure R6 to send twice as much traffic to R4.

- Only do this for IPv4 traffic. (It doesn’t work for IPv6).

Answer

# R3 router bgp 65000 add ipv4 maximum-paths ibgp 2 bgp dmzlink-bw # R4 router bgp 65000 template peer-policy IBGP_V4V6 send-community both exit-peer-policy add ipv4 bgp dmzlink-bw neighbor 10.4.6.6 dmzlink-bw # R5 router bgp 65000 template peer-policy IBGP_V4V6 send-community both exit-peer-policy add ipv4 bgp dmzlink-bw neighbor 10.5.6.6 dmzlink-bw # R6 router bgp 65006 add ipv4 bgp dmzlink-bw neighbor 10.4.6.4 dmzlink-bw neighbor 10.5.6.5 dmzlink-bw maximum-paths 2