JNCIE-SP MODULAR LAB

SYS-101

Initial System Settings

Task and Solution Guide

| PLATFORM vJunos-router 25.4 | DURATION 3 hours | TARGET 2 repetitions |
| --- | --- | --- |

Solution Guide — Revision 3

Platform: Containerlab, four vJunos-router 25.4R1.12 nodes, one consolidated services node

Estimated time: 3 hours | Difficulty: Intermediate | Target repetitions: 2

Lab purpose

Build and verify a secure management baseline on R1-R4. Containerlab/vrnetlab

injects management addressing, management-instance, mgmt_junos, and its

default route. Startup files deliberately do not configure those objects.

Addressing

| Node | Containerlab management IP | Loopback | Final hostname | Role |
| --- | --- | --- | --- | --- |
| R1 | 172.31.1.11/24 | 192.0.2.11/32 | R1 | Junos router |
| R2 | 172.31.1.12/24 | 192.0.2.12/32 | R2 | Junos router |
| R3 | 172.31.1.13/24 | 192.0.2.13/32 | R3 | Junos router |
| R4 | 172.31.1.14/24 | 192.0.2.14/32 | R4 | Junos router |
| S1 | 172.31.1.100/24 | — | S1 | DNS, NTP, RADIUS, Syslog and SCP archive |

Inside each vJunos VM, fxp0.0 is 10.0.0.15/24 and the management next hop

is 10.0.0.2. The 172.31.1.x addresses remain the Containerlab-facing

addresses used from the host and by S1.

Task 1 - Validate baseline and identity

Objective Confirm startup configuration loading, injected management connectivity, S1 reachability, and production identity.

Requirements

Deploy all five nodes and verify that they are operational.

Confirm Containerlab management addresses and the injected fxp0.0 configuration.

Change bootstrap hostnames to R1, R2, R3, and R4.

Save deployment and management-interface evidence.

Solution

Deploy and inspect the lab:

make build-services make deploy containerlab inspect --topo topology.clab.yml

On every router, replace <NODE> with its node name:

configure set system host-name <NODE> commit confirmed 5 run show interfaces terse fxp0 run ping 172.31.1.100 routing-instance mgmt_junos count 3 commit

Do not add management configuration to the startup file. An internal address

such as 10.0.0.15/24 is expected on fxp0.0; the host still reaches the node

through the Containerlab address.

Task 2 - Central and local Syslog

Objective Create troubleshooting evidence before implementing the remaining tasks.

Requirements

- jncie-sp-messages must record information and interactive commands.

- user-commands must record interactive commands.

- s1 (172.31.1.100) must receive notice messages and configuration changes through mgmt_junos.

- User ops must receive warning messages when that user is logged in.

- Archive three local files with a size of 100 KB each.

Solution

configure set system syslog archive size 100k set system syslog archive files 3 set system syslog user ops any warning set system syslog host 172.31.1.100 any notice set system syslog host 172.31.1.100 change-log any set system syslog host 172.31.1.100 routing-instance mgmt_junos set system syslog file jncie-sp-messages any info set system syslog file jncie-sp-messages interactive-commands any set system syslog file user-commands interactive-commands any commit

The ops target is valid before Task 7 creates the corresponding login template. It becomes useful whenever that identity is logged in.

Verify locally and on S1:

show configuration system syslog ¦ display set show log jncie-sp-messages ¦ last 20 show log user-commands ¦ last 20

find evidence/services -name '*.log' -size +0c -ls

Task 3 - Management services and retained route

Objective Enable required management protocols and preserve a remote management route if RPD is unavailable.

Requirements

- Enable SSH, Telnet, and FTP.

- Install static route 10.10.10.0/24 through 10.0.0.2 in mgmt_junos.

- Retain the route if RPD stops.

- Prevent the route from being advertised by dynamic routing protocols.

Solution

configure set system services ssh set system services telnet set system services ftp set routing-instances mgmt_junos routing-options static route 10.10.10.0/24 next-hop 10.0.0.2 set routing-instances mgmt_junos routing-options static route 10.10.10.0/24 retain set routing-instances mgmt_junos routing-options static route 10.10.10.0/24 no-readvertise commit confirmed 5 run show route table mgmt_junos.inet.0 10.10.10.0/24 exact detail commit

retain preserves forwarding state when RPD is unavailable;

no-readvertise prevents redistribution into dynamic routing protocols.

Task 4 - DNS and time zone

Objective Provide consistent name resolution and administrative time display.

Requirements

- Use S1 (172.31.1.100) as the DNS server through mgmt_junos.

- Use domain sys101.lab.

- Set Asia/Jakarta as the time zone.

- Resolve and reach s1.sys101.lab through mgmt_junos.

Solution

configure set system name-server 172.31.1.100 routing-instance mgmt_junos set system domain-name sys101.lab set system time-zone Asia/Jakarta commit run ping s1.sys101.lab routing-instance mgmt_junos count 3

The expected DNS answer is 172.31.1.100. If an older address is returned, rebuild S1 without cache and redeploy it.

Task 5 - Authenticated NTP

Objective Synchronize every router with S1 using authenticated NTP.

Requirements

- Configure the node-specific loopback address shown in the addressing table.

- Use S1 as the operational NTP server through mgmt_junos.

- Use MD5 key ID 1 and key value workbook.

- Trust key ID 1 and associate it with S1.

- Demonstrate an authenticated, reachable NTP association and synchronized status.

Solution

On each router, replace <LOOPBACK> with the address from the table:

configure set interfaces lo0 unit 0 family inet address <LOOPBACK> set system ntp authentication-key 1 type md5 value workbook set system ntp trusted-key 1 set system ntp server 172.31.1.100 key 1 routing-instance mgmt_junos commit

Use workbook for auth key value. The loopback supplies a usable source identity for this platform combination.

show configuration system ntp ¦ display set ¦ except value show ntp associations show ntp status

Wait for nonzero reach and sync_ntp. The association should show SKEY.

Task 6 - Configuration archival over SCP

Objective Archive configuration automatically after every commit.

Requirements

- Use S1 (172.31.1.100) SCP with lab / lab123 through mgmt_junos.

- Store files in /tmp on S1.

- Configure transfer on commit.

- Trigger at least one post-configuration archive.

- Demonstrate a non-empty archived configuration on S1.

Solution

configure set system archival configuration transfer-on-commit set system archival configuration routing-instance mgmt_junos set system archival configuration archive-sites "scp://lab@172.31.1.100/srv/ftp/archive" password lab123 commit

Verify:

show configuration system archival show log jncie-sp-messages ¦ match transfer show log jncie-sp-messages ¦ match conf.gz

find services/archive -type f -size +0c -ls

The tested vJunos FTP client rejects the internal routing-instance option. SCP is therefore the supported lab method. For manual testing:

file copy /config/juniper.conf.gz scp://lab@172.31.1.100/srv/ftp/archive/R1-test.conf.gz routing-instance mgmt_junos

Task 7 - Authorization classes and RADIUS mappings

Objective Map RADIUS identities to distinct Junos authorization profiles.

Requirements

- ops-class must have clear, network, reset, trace, and view permissions.

- noc-class must have all permissions but deny clear, configure, edit, and start-shell commands.

- Create local passwordless templates radius-ops and radius-noc for the two classes.

- Create local passwordless template remote with read-only authorization.

- Map RADIUS users ops and noc to their corresponding local templates with Juniper-Local-User-Name.

- Demonstrate permitted and denied actions for each constrained role and read-only authorization for an unmapped RADIUS user.

Solution

configure set system login class ops-class permissions clear set system login class ops-class permissions network set system login class ops-class permissions reset set system login class ops-class permissions trace set system login class ops-class permissions view set system login class noc-class permissions all set system login class noc-class deny-commands "(clear¦configure¦edit¦start shell)" set system login user radius-ops class ops-class set system login user radius-noc class noc-class set system login user remote class read-only commit

These are passwordless local authorization templates, not duplicate local credentials. S1 returns these FreeRADIUS attributes:

ops Cleartext-Password := "ops123" Juniper-Local-User-Name := "radius-ops" noc Cleartext-Password := "noc123" Juniper-Local-User-Name := "radius-noc" readonly Cleartext-Password := "readonly123"

The unmapped readonly identity inherits the special remote template. Test both permitted and denied commands after Task 8 activates RADIUS.

Task 8 - RADIUS authentication with local fallback

Objective Use centralized authentication first while maintaining local recovery access.

Requirements

- Authentication order must be RADIUS followed by local password.

- Use S1 (173.31.1.100) through mgmt_junos with shared secret workbook.

- Set retry to 1 and timeout to 2 seconds.

- Preserve local break-glass user lab / lab123.

- Prove RADIUS authentication while S1 is available (ops / ops123 ; noc / noc123).

- Prove local fallback while S1 RADIUS is unavailable without causing administrative lockout.

Solution

Keep the current session open:

configure set system authentication-order radius set system authentication-order password set system radius-server 172.31.1.100 secret workbook set system radius-server 172.31.1.100 routing-instance mgmt_junos set system radius-server 172.31.1.100 retry 1 set system radius-server 172.31.1.100 timeout 2 commit confirmed 10

Prove login as ops, noc, and readonly from a second terminal. Confirm the commit only after successful tests.

To prove local fallback, temporarily stop RADIUS without removing S1:

docker exec S1 supervisorctl stop radius ssh lab@R3 docker exec S1 supervisorctl start radius

lab / lab123 must work while RADIUS is stopped. This proves password fallback;

a normal local login while RADIUS is reachable does not prove which method was

used. Verify RADIUS traffic/logs on S1 when needed.

Final verification

bash scripts/verify.sh

Required final line:

RESULT: PASS

Rollback and reset

make destroy make deploy

enforce-startup-config: true reloads the supplied bootstrap baseline on a

clean deployment; Containerlab injects the management plane again.